System Dependability Lab

Exercises on Safety Assessment of Static Systems

Guohao DAI

Friday 28th October, 2022

DDL: 2022/11/19 00:00

1. Introduction

I have studied and compared three Computing Platform Designs that should support three applications (A1, A2 and A3). Each application Ai is implemented by two tasks AiL and AiR. The application Ai fails if both tasks AiL and AiR fail. A task fails if all the computers that can host it fail. We are interested in the following Failure Conditions:

FCAi loss of application Ai , with i∈{1,2,3}.

FC_One_Appli loss of at least one application.

All the FC are classified CATASTROPHIC for an operation time of T=103h.

Question 1.	What are the qualitative and quantitative safety requirements associated to the FCs?

Response:	The reason of all the FC are classified CATASTROPHIC, the qualitative and quantitative safety requirements associated to the FCs are:

| Criticality | Qualitative requirement | Quantitative requirement |
| --- | --- | --- |
| CATASTROPHIC | order ≥2 | Λ≤10-9 / flight hour |

2. Computing Platform Design – solution 1

Figure 1 presents the first solution for the computer platform design. In this solution the application fails if its computer fails. We assume that the loss of a computer is modelled by an exponential distribution of failure rate λ=10-5.h-1.

Question 2

1. Create a new file and build the fault-tree for the failure conditions FCAi and FC_One_Appli.

2. Compute the Minimal Cut Sets for FCAi and FC_One_Appli.

(1) The Minimal Cut Sets for FC_One_Appli:

| No. | Quantity | Probability | Percent | Events |  |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 9.90058e-05 | 0.333333 | C3L | C3R |
| 2 | 2 | 9.90058e-05 | 0.333333 | C2L | C2R |
| 3 | 2 | 9.90058e-05 | 0.333333 | C1L | C1R |

(2) The Minimal Cut Sets for FCA1:

| No. | Quantity | Probability | Percent | Events |  |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 9.90058e-05 | 1 | C1L | C1R |

(3) The Minimal Cut Sets for FCA2:

| No. | Quantity | Probability | Percent | Events |  |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 9.90058e-05 | 1 | C2L | C2R |

(4) The Minimal Cut Sets for FCA3:

| No. | Quantity | Probability | Percent | Events |  |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 9.90058e-05 | 1 | C3L | C3R |

3. Compute the mean failure rate of FCAi and FC_One_Appli.

(1) The mean failure rate of FC_One_Appli is:

ΛOne_Appli=PFCA1+PFCA2+P(FCA3)T=3×9.90×10-5103≈3×10-7

(2) The mean failure rate of FCAi is:

ΛFCA1=PFCA1T≈9.90×10-5103=9.90×10-8≈10-7

ΛFCA2=PFCA2T≈9.90×10-5103=9.90×10-8≈10-7

ΛFCA3=PFCA3T≈9.90×10-5103=9.90×10-8≈10-7

4. Are the Qualitative and Quantitative requirements enforced for failure conditions FCAi and FC_One_Appli? Justify the answer.

Response:

The Qualitative and Quantitative requirements are not enforced for failure conditions FCAi and FC_One_Appli. Because the order of each FC equals 2 (=2) and mean failure rate Λ is more than 10-9 (≥10-9).

3. Computing Platform Design – solution 2

Figure 2 describes the solution 2 for the computing platform design. In this solution the application fails if its computer fails except for task A1L (resp. A3R) that fails if both the computers C1L and C1Lb (resp. C3R and C3Rb) fail.

Question 3

1. Create a new file and build the fault-tree for the failure conditions FCAi and FC_One_Appli.

2. Compute the Minimal Cut Sets for FCAi and FC_One_Appli.

(1) The Minimal Cut Sets for FC_One_Appli:

| No. | Quantity | Probability | Percent | Events |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 9.90058e-05 | 0.980488 | C2L | C2R |  |
| 2 | 3 | 9.85124e-07 | 0.00975602 | C3L | C3R_a | C3R_b |
| 3 | 3 | 9.85124e-07 | 0.00975602 | C1L_a | C1L_b | C1R |

(2) The Minimal Cut Sets for FCA1:

| No. | Quantity | Probability | Percent | Events |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 3 | 9.85124e-07 | 1 | C1L_a | C1L_b | C1R |

(3) The Minimal Cut Sets for FCA2:

| No. | Quantity | Probability | Percent | Events |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 9.90058e-05 | 1 | C2L | C2R |  |

(4) The Minimal Cut Sets for FCA3:

| No. | Quantity | Probability | Percent | Events |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 3 | 9.85124e-07 | 1 | C3L | C3R_a | C3R_b |

3. Compute the mean failure rate of FCAi and FC_One_Appli.

(1) The mean failure rate of FC_One_Appli is:

ΛOne_Appli=PFCA1+PFCA2+P(FCA3)T≈PFCA2T≈10-7

(2) The mean failure rate of FCAi is:

ΛFCA1=PFCA1T≈9.85×10-7103=9.85×10-10≈10-9

ΛFCA2=PFCA2T≈9.90×10-5103=9.90×10-8≈10-7

ΛFCA3=PFCA3T≈9.85×10-7103=9.85×10-10≈10-9

4. Are the Qualitative and Quantitative requirements enforced for failure conditions FCAi and FC_One_Appli? Justify the answer.

Response:

- The Qualitative and Quantitative requirements are not enforced for failure conditions FCA2 and FC_One_Appli. Because the order of FCA2 and FC_One_Appli are both equal 2, moreover, their mean failure rate Λ is more than 10-9.

- The Qualitative and Quantitative requirements are enforced for failure conditions FCA1 and FCA3. Because the order of each FC equals 3 (≥2) and mean failure rate Λ is less than 10-9 (≤10-9).

4. Computing Platform Design – solution 3

The solution 3 of the computing platform design is described by the figure 3. In this solution the application fails if its computer fails and if the spare computer SpL (resp. SpR) cannot be used as a backup. The spare SpL (resp. SpR) can be used by:

- A1L (resp. A1R) if C1L (resp. C1R) fails,

- A2L (resp. A2R) if C2L (resp. C2R) fails and not used by A1L (resp. A1R),

- A3L (resp. A3R) if C3L (resp. C3R) fails and not used by A1L, or A2L (resp. A1R, or A2R).

Question 4.

1. Create a new file and build the fault-tree for the failure conditions FCAi and FC_One_Appli.

2. Compute the Minimal Cut Sets for FCAi and FC_One_Appli.

(1) The Minimal Cut Sets for FC_One_Appli:

| No. | Quantity | Probability | Percent | Events |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4 | 9.80215e-09 | 0.0714286 | C2L | C2R | SPL | SPR |
| 2 | 4 | 9.80215e-09 | 0.0714286 | C1R | C2L | C2R | SPL |
| 3 | 4 | 9.80215e-09 | 0.0714286 | C1L | C2L | C2R | SPR |
| 4 | 4 | 9.80215e-09 | 0.0714286 | C1L | C1R | C2L | C2R |
| 5 | 4 | 9.80215e-09 | 0.0714286 | C2L | C2R | C3L | C3R_a |
| 6 | 4 | 9.80215e-09 | 0.0714286 | C2R | C3L | C3R_a | SPL |
| 7 | 4 | 9.80215e-09 | 0.0714286 | C1L | C2R | C3L | C3R_a |
| 8 | 4 | 9.80215e-09 | 0.0714286 | C1L | C1R | SPL | SPR |
| 9 | 4 | 9.80215e-09 | 0.0714286 | C1L | C3L | C3R_a | SPR |
| 10 | 4 | 9.80215e-09 | 0.0714286 | C1L | C1R | C3L | C3R_a |
| 11 | 4 | 9.80215e-09 | 0.0714286 | C3L | C3R_a | SPL | SPR |
| 12 | 4 | 9.80215e-09 | 0.0714286 | C1R | C3L | C3R_a | SPL |
| 13 | 4 | 9.80215e-09 | 0.0714286 | C2L | C3L | C3R_a | SPR |
| 14 | 4 | 9.80215e-09 | 0.0714286 | C1R | C2L | C3L | C3R_a |

(2) The Minimal Cut Sets for FCA1:

| No. | Quantity | Probability | Percent | Events |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4 | 9.80215e-09 | 1 | C1L | C1R | SPL | SPR |

(3) The Minimal Cut Sets for FCA2:

| No. | Quantity | Probability | Percent | Events |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4 | 9.80215e-09 | 0.25 | C2L | C2R | SPL | SPR |
| 2 | 4 | 9.80215e-09 | 0.25 | C1L | C2L | C2R | SPR |
| 3 | 4 | 9.80215e-09 | 0.25 | C1R | C2L | C2R | SPL |
| 4 | 4 | 9.80215e-09 | 0.25 | C1L | C1R | C2L | C2R |

(4) The Minimal Cut Sets for FCA3:

| No. | Quantity | Probability | Percent | Events |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4 | 9.80215e-09 | 0.1111111 | C2L | C2R | C3L | C3R_a |
| 2 | 4 | 9.80215e-09 | 0.1111111 | C2R | C3L | C3R_a | SPL |
| 3 | 4 | 9.80215e-09 | 0.1111111 | C1L | C2R | C3L | C3R_a |
| 4 | 4 | 9.80215e-09 | 0.1111111 | C1L | C3L | C3R_a | SPR |
| 5 | 4 | 9.80215e-09 | 0.1111111 | C1L | C1R | C3L | C3R_a |
| 6 | 4 | 9.80215e-09 | 0.1111111 | C3L | C3R_a | SPL | SPR |
| 7 | 4 | 9.80215e-09 | 0.1111111 | C1R | C3L | C3R_a | SPL |
| 8 | 4 | 9.80215e-09 | 0.1111111 | C2L | C3L | C3R_a | SPR |
| 9 | 4 | 9.80215e-09 | 0.1111111 | C1R | C2L | C3L | C3R_a |

3. Compute the mean failure rate of FCAi and FC_One_Appli.

(1) The mean failure rate of FC_One_Appli is:

ΛOne_Appli=PFCA1+PFCA2+P(FCA3)T≈14×9.80×10-9T≈1.4×10-10

(2) The mean failure rate of FCAi is:

ΛFCA1=PFCA1T≈9.80×10-9103=9.80×10-12

ΛFCA2=PFCA2T≈4×9.80×10-9103=3.92×10-11

ΛFCA3=PFCA3T≈9×9.80×10-9103=8.82×10-11

4. Are the Qualitative and Quantitative requirements enforced for failure conditions FCAi and FC_One_Appli? Justify the answer.

Response:

The Qualitative and Quantitative requirements are enforced for failure conditions FCAi and FC_One_Appli. Because the order of each FC equals 4 (≥2) and mean failure rate Λ is less than 10-9 (≤10-9).

5. Computing Platform Design – DAL Allocation

The group of Basic Computers is independent from Spare Computers:

BasicComputers= C1L,C2L,C3L, C1Lb, C1R,C2R, C3R,C3Rb

SpareComputers= {SpL,SpR}

Within a group Basic or Spare, all computers are dependent.

Question 5 	Knowing the independent group, for each solution complete the DAL allocation table 1 to allocate a DAL to the computers of the platform.

| Severity (FC) | DAL(FC) | Acceptable Frequency (Order of Magnitude) |
| --- | --- | --- |
| CAT | A | 10-9 |
| HAZ | B | 10-7 |
| MAL | C | 10-5 |
| MIN | D | 10-3 |
| NSE | E | - |

Table – Link between severity and DAL

(1) The DAL allocation table of Solution 1:

| FC | Initial DAL | MCS | Components |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | C1L | C2L | C3L | C1R | C2R | C3R |
| FCA1 | A | {C1L,C1R} | ≥A | - | - | ≥A | - | - |
| FCA2 | A | {C2L,C2R} | - | ≥A | - | - | ≥A | - |
| FCA3 | A | {C3L,C3R} | - | - | ≥A | - | - | ≥A |
| FC_One_Appli | A | {C1L,C1R} | ≥A | - | - | ≥A | - | - |
|  |  | {C2L,C2R} | - | ≥A | - | - | ≥A | - |
|  |  | {C3L,C3R} | - | - | ≥A | - | - | ≥A |
| Final |  |  | ≥A | ≥A | ≥A | ≥A | ≥A | ≥A |

(2) The DAL allocation table of Solution 2:

| FC | Initial DAL | MCS | Components |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | C1L | C2L | C3L | C1L_b | C1R | C2R | C3R | C3R_b |
| FCA1 | A | {C1L,C1R, C1L_b} | ≥A | - | - | ≥A | ≥A | - | - | - |
| FCA2 | A | {C2L,C2R} | - | ≥A | - | - | - | ≥A | - | - |
| FCA3 | A | {C3L,C3R,C3R_b} | - | - | ≥A | - | - | - | ≥A | ≥A |
| FC_One_Appli | A | {C1L,C1R, C1L_b} | ≥A | - | - | ≥A | ≥A | - | - | - |
|  |  | {C2L,C2R} |  | ≥A | - | - | - | ≥A | - | - |
|  |  | {C3L,C3R, C3R_b} | - | - | ≥A | - | - | - | ≥A | ≥A |
| Final |  |  | ≥A | ≥A | ≥A | ≥A | ≥A | ≥A | ≥A | ≥A |

(3) The DAL allocation table of Solution 3:

| FC | Initial DAL | MCS | Components |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | C1L | C2L | C3L | C1R | C2R | C3R_a | SpL | SpR |
| FCA1 | A | {C1L,C1R, SpL,SpR} | ≥A | - | - | ≥A | - | - | ≥C | ≥C |
| FCA2 | A | {C2L,C2R, SpL,SpR} | - | ≥A | - | - | ≥A | - | ≥C | ≥C |
|  |  | {C1L,C2L, C2R,SpR} | ≥A | ≥A | - | - | ≥A | - | - | ≥C |
|  |  | {C1R,C2L, C2R,SpL} | - | ≥A | - | ≥A | ≥A | - | ≥C | - |
|  |  | {C1L,C1R, C2L,C2R} | ≥A | ≥A | - | ≥A | ≥A | - | - | - |
| FCA3 | A | {C2L,C2R, C3L,C3R_a} | - | ≥A | ≥A | - | ≥A | ≥A | - | - |
|  |  | {C2R,C3L, C3R_a,SpL} | - | - | ≥A | - | ≥A | ≥A | ≥C | - |
|  |  | {C1L,C2R, C3L,C3R_a} | ≥A | - | ≥A | - | ≥A | ≥A | - | - |
|  |  | {C1L,C3L, C3R_a,SpR} | ≥A | - | ≥A | - | - | ≥A | - | ≥C |
|  |  | {C1L,C1R, C3L,C3R_a} | ≥A | - | ≥A | ≥A | - | ≥A | - | - |
|  |  | {C3L,C3R_a,SpL,SpR} | - | - | ≥A | - | - | ≥A | ≥C | ≥C |
|  |  | {C1R,C3L, C3R_a,SpL} | - | - | ≥A | ≥A | - | ≥A | ≥C | - |
|  |  | {C2L,C3L, C3R_a,SpR} | - | ≥A | ≥A | - | - | ≥A | - | ≥C |
|  |  | {C1R,C2L, C3L,C3R_a} | - | ≥A | ≥A | ≥A | - | ≥A | - | - |
| FC_One_Appli | A | {C2L,C2R, SpL,SpR} | - | ≥A | - | - | ≥A | - | ≥C | ≥C |
|  |  | {C1L,C2L, C2R,SpR} | ≥A | ≥A | - | - | ≥A | - | - | ≥C |
|  |  | {C1R,C2L, C2R,SpL} | - | ≥A | - | ≥A | ≥A | - | ≥C | - |
|  |  | {C1L,C1R, C2L,C2R} | ≥A | ≥A | - | ≥A | ≥A | - | - | - |
|  |  | {C2L,C2R, C3L,C3R_a} | - | ≥A | ≥A | - | ≥A | ≥A | - | - |
|  |  | {C2R,C3L, C3R_a,SpL} | - | - | ≥A | - | ≥A | ≥A | ≥C | - |
|  |  | {C1L,C2R, C3L,C3R_a} | ≥A | - | ≥A | - | ≥A | ≥A | - | - |
|  |  | {C1L,C1R, SpL,SpR} | ≥A | - | - | ≥A | - | - | ≥C | ≥C |
|  |  | {C1L,C3L, C3R_a,SpR} | ≥A | - | ≥A | - | - | ≥A | - | ≥C |
|  |  | {C1L,C1R, C3L,C3R_a} | ≥A | - | ≥A | ≥A | - | ≥A | - | - |
|  |  | {C3L,C3R_a,SpL,SpR} | - | - | ≥A | - | - | ≥A | ≥C | ≥C |
|  |  | {C1R,C3L, C3R_a,SpL} | - | - | ≥A | ≥A | - | ≥A | ≥C | - |
|  |  | {C2L,C3L, C3R_a,SpR} | - | ≥A | ≥A | - | - | ≥A | - | ≥C |
|  |  | {C1R,C2L, C3L,C3R_a} | - | ≥A | ≥A | ≥A | - | ≥A | - | - |
| Final |  |  | ≥A | ≥A | ≥A | ≥A | ≥A | ≥A | ≥C | ≥C |

6. Computing Platform Design – Failed components

It is not possible to repair failed components in any airport so it should be possible to fly the aircraft safely with some components failed.

Question 6	Duplicate the table 2 in your report and complete:

- The first one considering the qualitative requirement (i.e. satisfy FC_One_appli i order bound);

Response: if (number of components >2) then (“OK”) else (“KO”)

| Solution | components |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | C1L | C2L | C3L | C1L_b | C1R | C2R | C3R / C3R_a | C3R_b | SpL | SpR |
| 1 | KO | KO | KO | - | KO | KO | KO | - | - | - |
| 2 | OK | KO | OK | OK | OK | KO | KO | OK | - | - |
| 3 | OK | OK | OK | - | OK | OK | OK | - | OK | OK |

- The second one considering the qualitative requirement (i.e. satisfy FC_One_appli i mean failure rate bound).

Response: if (mean failure rate Λ≤10-9 ) then (“OK”) else (“KO”)

| Solution | components |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | C1L | C2L | C3L | C1L_b | C1R | C2R | C3R / C3R_a | C3R_b | SpL | SpR |
| 1 | KO | KO | KO | - | KO | KO | KO | - | - | - |
| 2 | KO | KO | KO | KO | KO | KO | KO | KO | - | - |
| 3 | KO | KO | KO | - | KO | KO | KO | - | KO | KO |

7. Computing Platform Design – Comparison

We suppose that the cost of a solution mainly depends on the number of computers and their associated DAL (i.e. costs are: DAL_A = 20, DAL_B = 15, DAL_C = 5; DAL_D = 4; DAL_E = 0).

Question 7 	Copy and complete the table 3 to compare the three solutions with respect to their cost, safety and its capability to fly with a faulty computer. What is your preferred solution? Can you imagine a better solution?

Response:

The table of solution comparison is shown as below:

| Solution | Fulfilled safety requirement |  | Acceptable with failed component | Cost |
| --- | --- | --- | --- | --- |
|  | Qualitative | Quantitative |  |  |
| 1 | OK | KO | KO | 120 |
| 2 | OK | KO | KO | 160 |
| 3 | OK | OK | KO | 130 |

To be continues