Second Labs on Real-Time Scheduling

Guohao DAI

November 25th, 2022

Exercise 1

Let’s assume the following task sharing resources R1, R2 and R3:

- The Simulation of this configuration without a specific protocol for resource allocation is not schedulable. Because T1 missed its deadline twice: (absolute deadline = 12; completion deadline = 17) and (absolute deadline = 32; completion deadline = 37).

- The Simulation of this configuration with the Priority Inheritance protocol for resource allocation is not schedulable. Because T1 and T2 both missed its deadline twice:

- T1: (absolute deadline = 12; completion deadline = 15) and (absolute deadline = 32; completion deadline = 35).

- T2: (absolute deadline = 15; completion deadline = 16) and (absolute deadline = 35; completion deadline = 36).

- The Simulation of this configuration with the Stack-based Protocol (Immediate Ceiling Inheritance Protocol) for resource allocation is schedulable.

Exercice 2

Let’s assume the following task sharing resources  R1, R2, R3 and R4:

- The Simulation of this task configuration is not schedulable because of every task’s WCET(capacity).

Exercice 3

- Is the following task configuration schedulable on one processor with two cores using a fully global Rate Monotonic scheduler?

- This task configuration schedulable on one processor with two cores is using a fully global Rate Monotonic scheduler: “No deadline missed in the computed scheduling: the task set is schedulable if we computed the scheduling on the feasibility interval. ”

- Is the following task configuration schedulable on one processor with two cores using a fully global Rate Monotonic scheduler?

- The simulation of this configuration with the Priority Inheritance protocol is not schedulable, because T3 missed its deadline three times: (absolute deadline = 12; completion deadline = 15), (absolute deadline = 24; completion deadline = 28) and (absolute deadline = 36; completion deadline = 43).

3． The conclusion is that if 2 tasks have the same periods and are synchronized, then for a dual-core processor, both cores will execute both tasks simultaneously. For the third task T3, it does not perform well. Compared to the second simulation, the first simulation performs task T3 better.

Exercice 4

Let’s assume the following configuration of tasks.

Exercice 5

Let’s assume the following configuration of tasks.

- Is it schedulable on three processors with a partitioned First-Fit Rate Monotonic scheduler?

- Same question with a partitioned First-Fit Earliest Deadline First scheduler.

ANSWER:

Firstly, we calculate the processor utilization rate of each task :

| Task 2 | U2=C2P2=48 | Task 4 | U4=C4P4=1220 | Task 1 | U1=C1P1=26 |
| --- | --- | --- | --- | --- | --- |
| Task 3 | U3=C3P3=310 | Task 5 | U5=C5P5=150 | Task 6 | U6=C6P6=2050 |
|  |  | Task 7 | U6=C6P6=5100 |  |  |
|  |  | Task 8 | U8=C8P8=1100 |  |  |
| U=Ui=0.8 |  | U=Ui=0.608 |  | U=Ui≈0.733 |  |
| Processor 1 |  | Processor 2 |  | Processor 3 |  |

The tasks are schedulable on three processors with a partitioned First-Fit Rate Monotonic scheduler and First-Fit Earliest Deadline First scheduler.

First-Fit Rate Monotonic scheduler:

First-Fit Earliest Deadline First scheduler: